# UTC-PD Model Equations (Baseline)

## 1. Photocurrent transfer function — rigorous Ramo Jtot integral

Hph(ω)=1WnormH1+H2+H3+H4

Term 1 — undepleted-absorber electron (diffusion; carries τA):

H1=WA1+jωτA⋅2+jωτR2 (1+jωτR)

Term 2 — the same electron drifting the depleted region (carries τA):

H2=WC1+jωτA sinc​ωτC2e−jωτC/2

Term 3 — depleted-absorber in-situ electron (no τA):

H3=WAd sinc​ωτeD2e−jωτeD/2

Term 4 — depleted-absorber in-situ hole (no τA):

H4=WAd sinc​ωτh2e−jωτh/2

with

Wnorm=WA+WC+2 WAd,  sinc(x)=sinxx

## 2. Transit times

τA=WA 2De3+lnpmaxpmin,  τC=WC​dzveE(z),  τeD=τgcc+12 τdep

τh=WAdvh,  τR=εσ

## 3. Circuit model

Input impedance and reflection coefficient:

Zin(ω)=jωLCPW+1 jωCCPW+1Rp+jωLRp+1Zdev ,  Zdev=Rs+jωLCPW2+1jωCj

S11(ω)=Zin−Z0Zin+Z0,  Z0=50 Ω

Normalized impedance:

znorm=ZinZ0=1+S111−S11

Transimpedance (photocurrent to load voltage):

Hckt(ω)=VRLIph=RL jωLCPW+RL  jωCj+YA (1+jωCjZs) ,  YA=jωCCPW+1Rp+jωLRp+1jωLCPW+RL

with Zs=Rs+jωLCPW2.

## 4. Total response and RF power

Htot(ω)=Hph(ω) Hckt(ω),  PRF(ω)=| Iph Hph(ω) Hckt(ω) |22 RL

## 5. Bias-dependent junction capacitance (C–V)

Cj(V)∝1Vbi+V  ⇒  Cj(−7 V)=131 fF, Cj(−5 V)=161 fF